Demonstration 1 (noise bursts around the head)

I could hear the noise bursts move around the head for most angles. Around +/- 90 degrees (mainly right/left) the effect was weaker, and the source seemed to change in strength more than in position. This fits the model: there the sound mostly differs between the ears in level, and the simple first-order filter only roughly copies how a real head shadows the sound. The model also cannot tell front from back, because it only uses the left–right angle, so a source in front and one behind get exactly the same filters. The rotation therefore sounds closer to a left–right sweep than a full circle.

Demonstration 2 (reflection from the front, and occlusion)

I did not hear a separate reflection. The reflection arrives so shortly after the direct sound, which is far too short to be heard as an echo. The sound seemed to not have a lot of directionality bet if I had to say it would be from in front of me. With the occlusion filter, the sound became duller and more bass-heavy, as if heard through an obstacle. The limitation here is that the occlusion is only a first-order low-pass filter. Real obstacles affect sound in a more complex way that depends on their size and shape, so this only approximates the muffled effect. Since everything comes from the front, the lack of difference in SPL for each ear also leaves the sound without a clear sense of distance or of being outside the head.

Demonstration 3 (reflection from the left)

The direct sound was heard from the front. The quieter reflection seemed to amplify the sound in a specific direction instead and made it seem to come slightly from the left. A limitation here is that the reflection is represented only by the simple delay and filter for a source at 90° to the left. A real head, torso and shoulders would shape the sound from the side in a more complex and individual way, and the model has no room or wall effects beyond the single reflection.